Helwan University                                                                  Irrigation Design (II)

Faculty of Engineering                                                              4 th Year Civil

Civil Engineering Department                                                   2012-2013

Sheet  (5)

1. What are the factors that govern the selection of a dam type?

2. What are the different types of reservoirs and clarify when we can

select of each type.

3. Define the flood Hydrograph & the Mass Flow Curve.

4. Why is a deep reservoir better than a shallow reservoir.

5. Show by Sketches the difference between inflow and outflow

hydrographs.

6. The construction cost for certain possible heights of a dam at a given

site have been estimated and are tabulated below. The storage

capacity corresponding to each height is also given.

Determine the most economical Dam height .

| Dam Height (m) | 5 | 10 | 20 | 30 | 40 | 50 | 60 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Construction Cost (mill. $) | 52 | 80 | 130 | 200 | 270 | 410 | 650 | 1600 |
| Storage Capacity (mill.m³) | 35 | 55 | 150 | 250 | 340 | 460 | 580 | 1100 |

7. The monthly rainfall records of a flood control reservoir  and the

required outflow from the same reservoir are

given in the following table

| Month | Sept. | Oct. | Nov. | Dec. | Jan. | Feb. | March | April | May | June | July | August |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Monthly Inflow X 10³( m³/s) | 2.1 | 2.1 | 2.4 | 10.4 | 24.6 | 36.8 | 33 | 32.5 | 65.8 | 57.2 | 8.6 | 4.3 |
| Monthly Outflow X 10³( m³/s) | 20 | 20 | 20 | 24 | 24 | 30 | 25 | 25 | 30 | 25 | 20 | 16.8 |

It is required to:

a) Plot the inflow hydrograph.

b) Determine the maximum reservoir storage.

8. The annual runoff reaching the site of the over-year storage reservoir

behind a gravity dam is as follows:

| Year | 1972 | 1973 | 1974 | 1975 | 1976 | 1977 | 1978 | 1979 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Runoff (cm) | 98 | 143.5 | 168.3 | 94 | 95.3 | 152.4 | 110 | 131.3 |

If the catchment area of the reservoir is 1675 km² , It is required to:

- Plot the mass inflow curve.

- Determine the average yield from the catchment area.

- Determine the maximum reservoir storage capacity

if the average annual demand is 2079x106 m3  .

- In which year(s) is the reservoir full .

- Determine the spillwy capacity if it is founded.

9.   A masonry gravity dam is 94m high and has  a crest width of 9m.

The US face of the dam has a constant inclination of  10o  to the vertical .

The dam  is designed to sustain maximum reservoir water depth of 89m,

while no tail water exists at the downstream face of the dam.

If the dam has the following data at the site:

Depth of accumulated silt at bottom of US face of dam = 12m

Wind speed = 80km/hr and fetch = 40km

Max. expected ice thickness due to frost action at water surface = 1.5m

Acceleration of earthquake force:

horizontal =(1/10) g &vertical =(1/12) g

Specific weight of dam material = 2.2 t/m3

Max. all. Stress of rock foundation at site = 120 t/m2

It is required to:

- Define the type of dam according to height .

- Determine all forces acting on dam.

- Check the dam stability against overturning, overstressing and sliding.